SIMATS ENGINEERING

ASSESSMENT TOOL 1

COURSE NAME:	CSA07 COURSE CODE: Computer Networks

COURSE OUTCOME COVERED

CO1: Apply the functions of OSI layer in enabling reliable data transmission across networks. (BL2, BL3)

Assessment Tool Weightage: 40%

Annexure A

SHORT ANSWER TEST

Dr.V.Parthipan

Code of Conduct:

I ARASAVILLI KARTHEEK(192565065) certify that this submission is my original work and that I have adhered to the guidelines specified for this assessment.

I understand that any violation of academic integrity rules will result in disciplinary action.

Signature of the student:

Arasavilli kartheek

Short Answer Test

1. A company needs to transfer a 150 MB design file from a client computer to a remote server over a network with a bandwidth of 50 Mbps. The Transport Layer divides the file into 1500-byte segments, while the Data Link Layer encapsulates each segment into frames by adding 40 bytes of header and trailer. Calculate the total number of segments and frames transmitted, the total transmission overhead introduced by the Data Link Layer, and estimate the total transmission time (ignoring propagation delay). Explain how the Transport and Data Link layers ensure reliable communication.

2. A Transport Layer protocol uses a sliding window size of 6 frames. Each frame carries 1024 bytes of user data. If the sender transmits 48 frames, calculate the number of transmission windows required. If one frame in every window is lost and must be retransmitted, determine the total number of retransmissions. Explain how flow control improves communication efficiency.

3. A 12,000-byte IP packet is transmitted through a router where the Maximum Transmission Unit (MTU) is 1500 bytes. Assuming each fragment requires a 20-byte IP header, calculate the total number of fragments created and the total header overhead introduced. Explain how the Network Layer ensures successful delivery of fragmented packets to the destination.

4. A Transport Layer protocol uses a sliding window size of 6 frames. Each frame carries 1024 bytes of user data. If the sender transmits 48 frames, calculate the number of transmission windows required. If one frame in every window is lost and must be retransmitted, determine the total number of retransmissions. Explain how flow control improves communication efficiency.

5. A multimedia application transfers a 600 MB video file. The Session Layer inserts a synchronization checkpoint after every 60 MB of transmitted data. During transmission, a failure occurs after 390 MB has been transferred. Determine the number of checkpoints created before failure and calculate the amount of data that must be retransmitted after recovery. Explain the importance of synchronization in the Session Layer.

6. Before transmission, the Presentation Layer compresses a 900 MB file by 40%. The compressed data is encrypted, adding 5% overhead to the compressed size. Calculate the compressed file size, the encrypted file size, and the total percentage reduction compared to the original file. Explain how the Presentation Layer improves transmission efficiency and security.

7.An FTP application transfers 3 GB of data over a network with an effective throughput of 120 Mbps. The Application Layer divides the data into requests of 3 MB each. Calculate the total number of requests generated and estimate the total transmission time in seconds. Discuss how the Application Layer provides reliable services to users.

8.A packet experiences the following processing delays at different OSI layers: Application Layer = 4 ms, Transport Layer = 3 ms, Network Layer = 5 ms, Data Link Layer = 2 ms, and Physical Layer = 1 ms. The propagation delay is 18 ms, and the transmission delay is 12 ms. Calculate the total end-to-end delay experienced by the packet and explain how each layer contributes to the communication process.

9.A network transfers 80 MB of useful data using frames of 1600 bytes, where each frame contains 80 bytes of protocol overhead added by different OSI layers. Calculate the total number of frames required, the total overhead transmitted, and the transmission efficiency as a percentage. Explain how protocol overhead affects network performance.

10.A hospital transfers a patient's medical record of 240 MB through an IoT-enabled healthcare network. The Presentation Layer compresses the data by 30%, the Transport Layer divides the compressed data into 1400-byte segments, and the Data Link Layer adds 60 bytes of overhead to each frame. Calculate the compressed file size, the total number of segments transmitted, the total protocol overhead introduced, and the final amount of data transmitted across the Physical Layer. Explain how multiple OSI layers work together to ensure secure and reliable communication.

# Computer Networks – Short Answer Test
Detailed Step-by-Step Solutions

Q1. File Transfer

Given:
File size = 150 MB = 150,000,000 bytes
Segment size = 1500 bytes
DL overhead/frame = 40 bytes
Bandwidth = 50 Mbps

Number of segments = 150,000,000 / 1500 = 100,000 segments
Number of frames = 100,000 frames

Total overhead = 100,000 × 40 = 4,000,000 bytes = 4 MB

Total transmitted data = 150 MB + 4 MB = 154 MB

Convert to bits:
154 MB × 8 = 1232 Mb

Transmission time = 1232 / 50 = 24.64 s

Answer:
Segments = 100,000
Frames = 100,000
Overhead = 4 MB
Transmission Time = 24.64 s

Transport Layer provides segmentation, flow control and error recovery.
Data Link Layer provides framing and error detection.

Q2. Sliding Window

Given:
Window size = 6 frames
Total frames = 48

Number of windows = 48 / 6 = 8

One frame lost in every window.

Retransmissions = 8 × 1 = 8

Answer:
Windows required = 8
Retransmissions = 8

Flow control prevents receiver overload and improves efficiency.

Q3. IP Fragmentation

Given:
Packet size = 12,000 bytes
MTU = 1500 bytes
IP header = 20 bytes

Payload per fragment = 1500 − 20 = 1480 bytes

Fragments = Ceiling(12000 / 1480)
= Ceiling(8.108)
= 9 fragments

Header overhead = 9 × 20
= 180 bytes

Answer:
Fragments = 9
Header overhead = 180 bytes

Network layer performs fragmentation, routing and reassembly.

Q4. Sliding Window (Same as Q2)

Windows = 48 / 6 = 8
Retransmissions = 8

Answer:
Windows required = 8
Retransmissions = 8

Flow control regulates transmission rate and avoids congestion.

Q5. Session Layer Synchronization

Given:
Video size = 600 MB
Checkpoint every 60 MB
Failure after 390 MB

Checkpoints:
60, 120, 180, 240, 300, 360

Number of checkpoints before failure = 6

Data retransmitted after recovery
= 390 − 360
= 30 MB

Answer:
Checkpoints = 6
Retransmitted data = 30 MB

Synchronization enables restarting from the latest checkpoint.

Q6. Compression and Encryption

Given:
Original file = 900 MB
Compression = 40%
Encryption overhead = 5%

Compressed size
= 900 × (1 − 0.40)
= 540 MB

Encrypted size
= 540 × 1.05
= 567 MB

Percentage reduction
= ((900 − 567)/900) × 100
= 37%

Answer:
Compressed size = 540 MB
Encrypted size = 567 MB
Reduction = 37%

Presentation Layer improves efficiency and security.

Q7. FTP Transfer

Given:
Data = 3 GB = 3000 MB
Request size = 3 MB
Throughput = 120 Mbps

Requests generated
= 3000 / 3
= 1000 requests

Data in bits
= 3000 × 8
= 24,000 Mb

Transmission time
= 24,000 / 120
= 200 s

Answer:
Requests = 1000
Transmission time = 200 s

Application layer provides FTP services to users.

Q8. End-to-End Delay

Processing delays:
Application = 4 ms
Transport = 3 ms
Network = 5 ms
Data Link = 2 ms
Physical = 1 ms

Total processing delay
= 4 + 3 + 5 + 2 + 1
= 15 ms

Propagation delay = 18 ms
Transmission delay = 12 ms

Total delay
= 15 + 18 + 12
= 45 ms

Answer:
End-to-end delay = 45 ms

Q9. Protocol Overhead

Given:
Useful data = 80 MB = 80,000,000 bytes
Frame size = 1600 bytes
Overhead = 80 bytes

Useful payload/frame
= 1600 − 80
= 1520 bytes

Frames required
= 80,000,000 / 1520
≈ 52,632 frames

Total overhead
= 52,632 × 80
= 4,210,560 bytes

Efficiency
= (1520 / 1600) × 100
= 95%

Answer:
Frames = 52,632
Overhead = 4,210,560 bytes
Efficiency = 95%

Q10. Hospital Medical Record Transfer

Given:
Original file = 240 MB
Compression = 30%
Segment size = 1400 bytes
Overhead/frame = 60 bytes

Compressed size
= 240 × 0.70
= 168 MB

Compressed data
= 168,000,000 bytes

Segments
= 168,000,000 / 1400
= 120,000

Protocol overhead
= 120,000 × 60
= 7,200,000 bytes
= 7.2 MB

Final transmitted data
= 168 + 7.2
= 175.2 MB

Answer:
Compressed size = 168 MB
Segments = 120,000
Overhead = 7.2 MB
Final transmitted data = 175.2 MB

OSI layers work together to provide compression, segmentation, framing, reliability and security.